Durability by Design

Engineering Design

# Name: ___________________________________			Date: __________________

Note

- Download the ‘Working Files’ folder for this hands-on practice. - Refer to the video tutorials linked within the worksheet. - Complete the tasks by answering the questions. - For guidance on installation, or if you encounter any difficulties along the way, please refer to the Installation and Troubleshooting Guide.

| Step 1: Accessing the ‘Working Files’ |  |
| --- | --- |
| Instructions | Reference |
| To import and pre-designed laptop assembly, refer to the following steps: - Open Autodesk Fusion 360, go to Files > Open. Expand the File menu to <create>, <open>, <upload>, <save>, <export>, <recover>, <3D print>, and <share> designs. - Choose open from my computer, Select the .f3d file that is now saved in your downloads folder. - Once the file has opened, you can see the assembly of a laptop and a ground. | Image: Opening a File Image: Preview of the Laptop Assembly |

Task 1

Review the working file, which showcases a laptop model. Note down your observations with regard to its design, appearance, and its ability to withstand a drop test. ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| Step 2: Modifying the Laptop Design for Ruggedness |  |
| --- | --- |
| Instructions | Reference |
| To create sketches, refer to the following steps: - Click Create Sketch. - Select the initial plane or face to begin the sketch on. - Let’s start by creating a simple line. Continue moving and clicking till you get a desired L shape. - Click Finish Sketch once you are done sketching your design. - Once finished, the sketch can now be seen in the workspace and is added to the browser as well. Note: Watch this video tutorial for a detailed explanation on how to create sketches in Fusion. | Image: Creating Sketches Image: Creating a Line Image: Creating an Enclosed L Shape. |
| To extrude or give depth to your sketch, refer to the following steps. - On the Design workspace toolbar, click Solid > Create > Extrude. Select the sketch profile you want to extrude. - Specify a Distance in the dialog. - When you click OK, you can see the new solid body on the canvas. - The body is also added to the Browser, within the Bodies folder. Note: Watch this video tutorial for a detailed explanation on how to create extrude sketches to create solid bodies. | Image: Accessing the Extrude Feature Image: Extruding the Sktech Image: Creating a New Body |

Task 2

How do you think incorporating an L-shaped body contributes to making the design more rugged? Where do you think this L-shaped body could be effectively attached to the laptop to contribute to its ruggedness? ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| Step 3: Assembling the Parts and Components |  |
| --- | --- |
| Instructions | Reference |
| To move a body/component, refer to the following steps: - Select Move/Copy under the Modify menu. - Select Body or component to be moved. - Select move type, such as free move to freely move the object in the workspace. - Input the move data or distance and click ok. | Image: Accessing the Move/Copy Tab Image: Moving a Body |
| To copy a body/component, refer to the following steps: - Follow the steps above. - Check the "Create Copy" box to create a copy of the original object in the new location. - This copy can be seen under the browser. | Image: Copying a Body |
| To attach bodies, refer to the following steps: - To attach bodies, select move type as “Point to Point”. - Select the body to be moved. - Input the origin and target points to be attached in both the bodies/components. - Once it is attached properly, click ok. | Image: Attaching Bodies |
| To combine solid bodies, refer to the following steps: - On the Solid tab>Modify Group, click Combine. Select the bodies to combine. - Join combines solid bodies into a single solid body. Note: Watch this video tutorial for a detailed explanation on how to combine solid bodies. | Image: Accessing the Combine Tool Image: Joining/Combining Bodies Image: Single Body |

| Note: Refer to this list of shortcuts, and their functions to further modify your design. |  |
| --- | --- |
| Keyboard Shortcut | Function |
| E | Extrude |
| L | Line |
| Q | Press Pull |
| M | Move Tool |
| F | Fillet |
| A | Appearance |
| R | 2-point Rectangle |
| T | Trim |
| D | Sketch Dimension |

Task 3

Briefly describe the areas you selected for ruggedization and explain how the tools you used contributed to strengthening the ruggedness of your design. ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| Step 4: Setting up the Simulation |  |
| --- | --- |
| Instructions | Reference |
| To create a simulation study, refer to the following steps: - To switch workspaces, expand the Workspace menu. Select Simulation. - Select Dynamic Event Simulation, Click on create study. | Image: Accessing the Simulation Workspace Image: Creating a Study |
| To assign materials to the laptop assembly, refer to the following steps: - Click on the Study Materials tab and Choose All libraries to get a wider set of materials. - Select the materials needed for all the different components and bodies present. - Click on Ok to assign the properties to the components for the selected materials in the study. | Image: Studying Materials Image: Materials Library Image: Assigning Materials |
| Task 4: What materials did you assign to the different parts of your design, and what was your rationale for selecting these materials? __________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |

| Step 5: Defining Simulation Conditions |  |
| --- | --- |
| Instructions | Reference |
| To add structural constraints, refer to the following steps: - Click on the Structural Constraints tab and choose the bodies that will be fixed. - In our drop test, the laptop will be a freefall while the ground stays fixed. - Select all the six faces of the ground to fix it completely. - Click on ok | Image: Accessing the Structural Constraints Tab Image: Fixing the Ground for Drop Test |
| To add initial velocity for the drop, refer to the following steps - Click on the Initial Linear Velocity tab and choose the bodies that will be moving. - Select Angle delta under direction type and choose the ground as reference, this will ensure that the laptop falls directly on the ground. - Change the magnitude to ensure impact happens. Due to the nature of event simulation, the simulation needs to have a small duration (such as 0.001 seconds) - Click on Ok - Under the simulation model, open up the load case and check on gravity to ensure real life simulation results occur. | Image: Accessing the Initial Linear Velocity Tab Image: Selecting the Reference Image: Setting Conditions for Gravity |
| To add contacts, refer to the following steps: - Click on the Global contacts tab and select generate. - Open Manage Contacts, under contacts group. Select all the components and bodies of the laptop that will be interacting with ground. Choose ‘Bonded’ for them. - Click on ok. | Image: Accessing the Global Contacts Tab Image: Managing Contacts |

| Step 5: Running the Simulation |  |
| --- | --- |
| Instructions | Reference |
| - Check if there are any unsolvable errors before running the simulation. | Image: Accessing the pre-check tab. |
| To run the simulation, refer to following steps: - Click on the Solve tab and select solve 1 study. - Wait for the simulation to run and give the results. | Image: Accessing the Solve Tab Image: Solve Study |
| To analyze the simulation, refer to the following steps: - To analyse the simulation, you can move the slider to check the deformation as per the timeline. - The colour spectrum shows the range of the stress. Blue being the lowest and red being the highest. - Under the results tool, you can get an animated video of the simulation and the detailed report analysis. - Visibility of the ground/laptop can be toggled on/off based on where you want to analyze the impact/deformation. | Image: Accessing the Simulation Results |
| Refer to the image on the right to see the impact of the drop test before ruggedization. You will notice that majority of the stress/effect of the impact is absorbed by the base of the laptop, and moves up to the screen. | Image: Before Ruggedization |
| Refer to the image on the right to see the impact of the drop test after ruggedization. You will notice that majority of the stress/effect of the impact has been absorbed by the ruggedized corners and the screen is not impacted. | Image: After Ruggedization |

Task 5

Submit screenshots highlighting the durability of your design during the drop test. Note your observations and compare the simulation results after ruggedization with those from before. How did your design handle the impact? ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________